Jupiter Database

Technical Reference & Exploratory Analysis

Shah Bekhsh  ·  DTU MSc Thesis 2026  ·  Last updated: March 2026

Note: This document is a living reference. Update it as you learn more about the database. It serves two purposes: a day-to-day cheat sheet while building the system, and raw material for the Data chapter of the thesis report.

# 1. Database Overview

JupiterXL is Denmark's national database for groundwater and drinking water chemistry, maintained by the Geological Survey of Denmark and Greenland (GEUS). It is publicly available as a nightly export and serves citizens, municipalities, researchers, and regulatory authorities.

The version used in this project is the PCJupiterXL Plus export, downloaded on 26 March 2026. It was restored from a PostgreSQL custom-format backup (pg_dump -Fc) into a local PostgreSQL 16 instance.

| Property | Value |
| --- | --- |
| Database format | PostgreSQL 16 custom backup (.backup) |
| Compressed size | 3.7 GB (zip) |
| Uncompressed backup | 19.9 GB |
| Schema name | pcjupiterxlplusviews |
| Total tables | 276 |
| Boreholes | 477,105 |
| Groundwater chemistry samples | 508,413 |
| Groundwater chemistry analyses | 14,671,938 |
| Chemical compounds tracked | 3,026 |
| Data freshness | Nightly export — as recent as March 2026 |

# 2. Schema Structure

The 276 tables fall into a small number of functional groups. Understanding this grouping is essential before writing any queries or designing agent tools.

## 2.1 Table families

| Family | Prefix | # Tables | Description |
| --- | --- | --- | --- |
| Core borehole & location | (none) | ~8 | borehole, intake, screen, locality. The spine of the database. |
| Groundwater chemistry | grw* | 8 | grwchemsample, grwchemanalysis, grwsoilsample, grwairanalysis etc. |
| Drinking water plant chemistry | plt* | 8 | Same structure as grw* but measured at treatment plant outlets. |
| Point-of-interest chemistry | poi* | 8 | Chemistry at contaminated sites and monitoring points. |
| Drinking water infrastructure | drw* | 6 | drwplant, drwplantintake, drwfirm — water supply companies and plants. |
| Compound & limit lookups | (none) | 5 | compoundlist, compoundgroup, compoundgrouplist, limitlist. |
| Geology & lithology | (none) | ~8 | lithsamp, lithstra, biostra, minerals etc. |
| Water levels & hydrology | (none) | 6 | watlevel, wrrcatchment, wrrexport, wrrsupply. |
| Code lookup tables | code_* | ~170 | One table per coded field. e.g. code_752 = measurement units. |
| Data dictionary | jup* | 6 | juptable, jupfield, jupheadr — Jupiter's internal field-level metadata. |

Note: ~170 of the 276 tables are code_* lookup tables. The actual data model has only ~30 meaningful tables.

## 2.2 The code_* lookup system

Every coded (numeric) field in Jupiter references a code_* table via a codetype number. The codetype table maps codetype numbers to human-readable descriptions. Key codetypes for this project:

| Codetype | Table | Description | Used in |
| --- | --- | --- | --- |
| 752 | code_752 | Measurement units (mg/l, µg/l, pH etc.) — 292 units total | grwchemanalysis.unit, pltchemanalysis.unit, limitlist.unit |
| 46 | code_46 | Physical dimension units (mm, cm, inches) — for borehole diameters | borhdiam.unit, screen.unit, casing.unit |
| 221 | code_221 / compoundlist | Chemical compound numbers | grwchemanalysis.compoundno |
| 60 | code_60 | Municipality codes | Various location fields |
| 17 | code_17 | Borehole purpose (drinking water, monitoring, geotechnical etc.) | borehole.purpose |

# 3. Key Tables — Columns & Descriptions

## 3.1 borehole (477,105 rows)

The central table. Every other table links back here via boreholeid. Contains location, administrative, and metadata for each physical borehole.

| Column | Type | Description |
| --- | --- | --- |
| boreholeid | numeric | Primary key. Used as FK in all related tables. |
| boreholeno | text | Human-readable DGU borehole number (e.g. '186. 358') |
| latitude / longitude | numeric | WGS84 decimal degrees. Some older records are NULL. |
| xutm32euref89 / yutm32euref89 | numeric | UTM zone 32 EUREF89 — the modern Danish standard coordinate system. |
| municipal | text | Municipality name in Danish (e.g. 'København', 'Frederiksværk'). |
| status | text | Borehole status code. 'R' = Registered/Active. |
| drilldepth | numeric | Total drilled depth in metres. |
| purpose | numeric | Coded field → code_17. Borehole purpose (drinking water, monitoring etc.) |
| drilstdate / drilendate | timestamp | Drilling start and end dates. |

## 3.2 grwchemsample (508,413 rows)

One row per sampling event at a borehole. Links boreholes to their chemistry analyses. Note: contains both boreholeid AND intakeid directly, so the intake join can be skipped for basic queries.

| Column | Type | Description |
| --- | --- | --- |
| sampleid | numeric | Primary key. Referenced by grwchemanalysis.sampleid. |
| boreholeid | numeric | FK → borehole.boreholeid |
| intakeid | numeric | FK → intake.intakeid (depth interval within the borehole) |
| sampledate | timestamp | Date and time the sample was collected. |
| purpose | numeric | Coded field — reason for sampling. |
| watertype | numeric | Coded field — type of water sampled. |
| laboratory | numeric | Coded field → laboratory lookup. Which lab processed the sample. |
| qualitycontrol | numeric | Quality assurance status of the sample. |

## 3.3 grwchemanalysis (14,671,938 rows)

The core chemistry table. One row per individual compound measurement. This is where nitrate levels, PFAS concentrations, pH, and all other chemical parameters live.

| Column | Type | Description |
| --- | --- | --- |
| analysisid | numeric | Primary key. |
| sampleid | numeric | FK → grwchemsample.sampleid |
| compoundno | numeric | FK → compoundlist.compoundno. Which chemical was measured. |
| amount | numeric | The standardised measurement value. Always use this over reportedamount. |
| unit | numeric | FK → code_752. The unit of measurement (e.g. 1 = mg/l). |
| detectionlimit | numeric | Values below this threshold are below detection. amount=0 often means <DL. |
| quantificationlimit | numeric | Lower bound for reliable quantification. Added in 2024. |
| reportedamount / reportedunit | numeric | Raw values as submitted by the laboratory. Use amount/unit instead. |
| analysisdate | timestamp | Date the analysis was performed in the laboratory. |
| accreditedindicator | text | Whether the analysis was performed by an accredited laboratory. |

## 3.4 compoundlist (3,026 rows)

Lookup table for all 3,026 chemical compounds tracked in Jupiter. The key bridge between human-readable compound names and the numeric compoundno used throughout the analysis tables.

| Column | Type | Description |
| --- | --- | --- |
| compoundno | numeric | Primary key. Used as FK in all analysis tables. |
| long_text | text | Full compound name in Danish (e.g. 'Nitrat', 'Perfluorbutansulfonsyre'). |
| short_text | text | Abbreviated name (e.g. 'Nitrat'). Sometimes NULL. |
| casno | numeric | International CAS Registry Number for unambiguous identification. |
| grwunit | numeric | Default unit for groundwater measurements → code_752. |
| drwunit | numeric | Default unit for drinking water measurements → code_752. |
| molarweight | numeric | Molar weight in g/mol. |

## 3.5 compoundgrouplist & compoundgroup

Compounds are organised into groups (e.g. all PFAS compounds under one group). This is essential for queries like 'are there any PFAS in my water?' where you want to search across dozens of individual compounds at once.

| Table | Key columns | Description |
| --- | --- | --- |
| compoundgrouplist | compoundgroupno, longtext, shorttext | Group definitions. e.g. compoundgroupno=110, longtext='Perfluorerede stoffer' (PFAS). |
| compoundgroup | compoundno, compoundgroupno | Junction table linking individual compounds to their groups. Also contains conversion factors between compounds. |

Note: Key group IDs: compoundgroupno=110 = 'Perfluorerede stoffer' (all PFAS compounds). Query compoundgroup WHERE compoundgroupno=110 to get all PFAS compound numbers.

# 4. Core Query Patterns

These are the verified, working query patterns for the most common question types. All tested against the live JupiterXL database.

## 4.1 Standard chemistry query

The fundamental join chain: borehole → grwchemsample → grwchemanalysis → compoundlist → code_752 (units). Use this for any question about chemical measurements at boreholes.

SELECT

b.boreholeno,

b.municipal,

b.latitude,

b.longitude,

s.sampledate,

a.amount,

u.longtext AS unit,

c.long_text AS compound,

a.detectionlimit

FROM pcjupiterxlplusviews.borehole b

JOIN pcjupiterxlplusviews.grwchemsample s ON s.boreholeid = b.boreholeid

JOIN pcjupiterxlplusviews.grwchemanalysis a ON a.sampleid = s.sampleid

JOIN pcjupiterxlplusviews.compoundlist c ON c.compoundno = a.compoundno

JOIN pcjupiterxlplusviews.code_752 u ON u.code = a.unit

WHERE a.compoundno = 246          -- Nitrat

AND b.municipal = 'Frederiksværk'

AND s.sampledate >= '2020-01-01'

ORDER BY s.sampledate DESC

LIMIT 50;

## 4.2 Compound lookup by name

Use this before constructing chemistry queries when the user names a compound. The agent should always look up the compoundno first rather than hardcoding it.

SELECT compoundno, long_text, short_text, casno, grwunit

FROM pcjupiterxlplusviews.compoundlist

WHERE long_text ILIKE '%nitrat%'

OR short_text ILIKE '%nitrat%';

## 4.3 PFAS group query

To query all PFAS compounds at once, join through compoundgroup using the group number for 'Perfluorerede stoffer' (compoundgroupno = 110).

SELECT c.compoundno, c.long_text

FROM pcjupiterxlplusviews.compoundlist c

JOIN pcjupiterxlplusviews.compoundgroup cg ON cg.compoundno = c.compoundno

WHERE cg.compoundgroupno = 110;

Then use the returned compoundno values in an IN clause in the chemistry query:

WHERE a.compoundno IN (SELECT c.compoundno

FROM pcjupiterxlplusviews.compoundlist c

JOIN pcjupiterxlplusviews.compoundgroup cg ON cg.compoundno = c.compoundno

WHERE cg.compoundgroupno = 110)

## 4.4 Geographic proximity query

For 'boreholes near location X' queries. Requires the user to supply a lat/lon coordinate. Uses the Haversine approximation — accurate enough for Denmark at this scale.

SELECT boreholeid, boreholeno, municipal, latitude, longitude,

111.045 * DEGREES(ACOS(LEAST(1.0,

COS(RADIANS(b.latitude)) * COS(RADIANS(55.68)) *

COS(RADIANS(b.longitude) - RADIANS(12.57)) +

SIN(RADIANS(b.latitude)) * SIN(RADIANS(55.68))

))) AS distance_km

FROM pcjupiterxlplusviews.borehole b

WHERE latitude IS NOT NULL

HAVING distance_km < 5

ORDER BY distance_km;

Note: Replace 55.68 and 12.57 with the target coordinates. For production use, consider adding PostGIS for proper geospatial indexing.

# 5. Important Gotchas & Data Quality Notes

| Issue | Detail | How to handle |
| --- | --- | --- |
| amount = 0 ≠ zero | Historical records (pre-1990s) use amount=0 to mean below detection limit, not a true zero measurement. | Always check detectionlimit. If amount <= detectionlimit, treat as 'below detection'. |
| NULL coordinates | Some boreholes have NULL latitude/longitude, particularly older ones. Municipal name may still be present. | Filter with WHERE latitude IS NOT NULL for geographic queries. Use municipal for non-spatial filtering. |
| Danish municipality names | The municipal column uses Danish names with special characters: 'København', 'Ålbæk', 'Frederiksværk'. | Use ILIKE for case-insensitive matching. Agent must handle name variants. |
| reported* vs amount/unit | grwchemanalysis has both reportedamount/reportedunit and amount/unit. The latter are GEUS-standardised. | Always use amount and unit, never the reported* variants. |
| Compound name language | All compound names in compoundlist are in Danish. | Agent system prompt must account for this. Map English user queries to Danish compound names. |
| unit column type | grwchemanalysis.unit is numeric, matching code_752.code which is also numeric. No casting needed. | JOIN on u.code = a.unit directly — no ::text cast required. |
| Schema prefix required | All tables are in the pcjupiterxlplusviews schema, not public. | All queries must prefix table names: pcjupiterxlplusviews.borehole etc. |
| 14.6M analysis rows | grwchemanalysis is very large. Unfiltered queries will be slow. | Always filter by compoundno and date range before joining to borehole. Add indexes if needed. |

# 6. Key Compound Reference

Quick reference for the most commonly queried compounds. Verified against the live database.

| Compound | compoundno | CAS No. | EU Limit (drinking water) | Notes |
| --- | --- | --- | --- | --- |
| Nitrat | 246 | 14797558 | 50 mg/l | Most common query. Use unit 1 (mg/l). |
| Nitrat-N | 247 | — | 11.3 mg/l (as N) | Same as nitrat but expressed as nitrogen content. |
| Nitrit+Nitrat | 93 | — | — | Combined measurement. |
| PFAS group | → group 110 | — | 0.1 µg/l (sum of 20) | Use compoundgroupno=110 to get all individual PFAS compounds. |
| Perfluorerede stoffer | Group 110 | — | — | Danish term for PFAS. longtext in compoundgrouplist. |

# 7. Built-in Data Dictionary (jup* tables)

Jupiter contains its own internal metadata system in the jup* tables. This is extremely valuable for building the agent's system prompt programmatically.

| Table | Description | Key use for agent |
| --- | --- | --- |
| juptable | One row per data table. Contains tablename, original Danish Oracle name (corrtablna), and table type. | Maps English table names to their Danish original names and descriptions. |
| jupfield | One row per column in each table. Contains tablename, columnname, codetype (which code_* table it references), and original Danish column name. | Programmatically determine which code_* table to join for any coded field. |
| jupheadr / jupheade | Header/heading definitions for reports. | Less relevant for query generation. |
| juphiera | Table hierarchy definitions. | Understanding parent-child relationships between tables. |

Note: The jupfield table maps every column to its codetype. For example: jupfield WHERE tablename='GRWCHEMANALYSIS' AND columnname='UNIT' returns codetype=752, confirming that unit values are decoded via code_752.

# 8. Local Setup Reference

| Parameter | Value |
| --- | --- |
| WSL2 host | GEUS3211W10WS |
| PostgreSQL version | 16.13 (Ubuntu 16.13-0ubuntu0.24.04.1) |
| Database name | jupiterxl |
| DB user | pgmcp |
| DB password | pgmcp_dev |
| Schema | pcjupiterxlplusviews |
| Backup file location | ~/Thesis/data/pcjupiterxlpluspg_xl.backup |
| Restore command | pg_restore -h localhost -U pgmcp -d jupiterxl --no-owner --role=pgmcp -v pcjupiterxlpluspg_xl.backup |
| Start PostgreSQL | sudo service postgresql start |
| Connect to DB | psql -h localhost -U pgmcp -d jupiterxl |

# 9. Next Steps

- Adapt database.py from the toy project to connect to jupiterxl with schema pcjupiterxlplusviews

- Update server.py MCP tools to be Jupiter-aware (schema prefix, compound lookup tool, group lookup tool)

- Add a search_compound(name) tool to the MCP server so the agent can resolve compound names to IDs

- Investigate the limitlist table — it likely contains official Danish drinking water threshold values for comparison

- Check whether PostGIS is available for proper geospatial queries, or implement Haversine in SQL

- Profile query performance on grwchemanalysis — 14.6M rows may need indexes on compoundno and sampleid

- Explore the drwplant and drwplantintake tables to understand drinking water plant coverage